BLUESTAQ LTD

SPECTRE Operator Guide

Space Planning, Evaluation & Counter-Threat Response Engine

Comprehensive Operator Reference — v2.0.0

23

MANOEUVRE METHODS

22

GUIDE SECTIONS

28

GCAT DATASETS

5

TRAINING LEVELS

SECTION 1

Introduction to SPECTRE

SPECTRE — the Space Planning, Evaluation & Counter-Threat Response Engine — is a real-time orbital manoeuvre planning and space intelligence tool designed for space defence operators. It enables rapid assessment of intercept trajectories, conjunction analysis, adversary intent evaluation, and defensive repositioning of space assets, all from a single browser-based interface.

SPECTRE is built on a fully self-contained, pure-Python orbital mechanics engine with no dependency on external astrodynamics software. Operators can compute 23 distinct intercept and manoeuvre methods spanning classical transfers (Lambert, Hohmann, bi-elliptic), tactical manoeuvres (phasing, CW relative motion, plane changes, J2 drift, collision avoidance, evasion), advanced analysis (GEO drift, NMC safety ellipse, manoeuvre classification, detectability), and decision-support tools (intent prediction, intercept envelopes, stability assessment, fingerprinting, formation defence, orbital terrain mapping, min-time intercept).

Beyond manoeuvre planning, SPECTRE provides a suite of space intelligence capabilities: Historical Pattern of Life (PoL) analysis detects past manoeuvres and characterises operator behaviour from TLE history; NOTSO Correlation cross-references Notice to Space Operators messages against detected manoeuvres to build operator behavioural profiles; Photometry Analysis detects brightness anomalies from light-curve data that may indicate attitude changes or physical alterations. The Decision Engine applies game-theoretic scenario evaluation to support commander-level response selection, and the Training Environment provides a sandboxed gamified environment for operator skill development across five progressive levels.

Integration with the Unified Data Library (UDL) provides authoritative live TLE data from authorised source providers. The GCAT Browser gives instant access to 28 of Jonathan McDowell's General Catalogue datasets spanning the full history of orbital activity.

Key design principle: SPECTRE is designed for speed-of-relevance decision-making. All computations run in-browser with sub-second response times, enabling operators to evaluate multiple intercept options before a conjunction window closes. No external astrodynamics libraries are required — SPECTRE runs on any system with a modern browser and network access to the SPECTRE server.

SECTION 2

System Overview

Architecture

SPECTRE follows a server-side rendering architecture with HTMX for dynamic partial updates. The backend is built on FastAPI (Python 3.14+) with Jinja2 templates. All orbital mechanics computations are performed by the spectre.astro package — a pure-Python library providing classical transfer solvers (Lambert, Hohmann, bi-elliptic), tactical manoeuvre solvers (phasing, CW relative motion, plane change, J2 drift, collision avoidance, evasion), advanced analysis (GEO drift, NMC safety ellipse, manoeuvre classification, detectability), decision-support solvers (intent prediction, intercept envelope, stability analysis, fingerprinting, formation defence, orbital terrain, min-time intercept), and SGP4-based orbit propagation.

Technology Stack

| Component | Technology | Purpose |
| --- | --- | --- |
| Web Framework | FastAPI + Uvicorn | Async HTTP server with ASGI |
| Templates | Jinja2 + HTMX | Server-side rendering with partial swaps |
| Orbit Propagation | SGP4 (via sgp4 library) | TLE-based satellite position/velocity |
| Classical Solvers | spectre.astro.maneuvers | Lambert, Hohmann, bi-elliptic transfers |
| Tactical Solvers | spectre.astro.tactical | Phasing, CW, plane change, J2 drift, COLA |
| Event Detection | spectre.astro.events | Apogee, perigee, node crossings |
| Authentication | SQLAlchemy + bcrypt | Operator credential management |
| TLE Source | UDL (Unified Data Library) | Authoritative TLE retrieval via HTTPS |
| Charting | Chart.js 4.x | Trade-space scatter plots in the browser |
| Design System | Bluestaq Dark Ops | Navy/gold operational colour scheme |

SECTION 3

Getting Started

Login

Navigate to the SPECTRE URL in your browser. Enter your operator username and password on the login screen. Credentials are managed by the system administrator and stored as bcrypt hashes — plaintext passwords are never retained.

UDL Credentials

To fetch authoritative TLE data from the Unified Data Library, connect your UDL account via the connection chip in the navigation bar. Click the UDL chip, enter your UDL username and password, and press Connect. A green status dot confirms a successful connection.

UDL credentials are held in session memory only and are never persisted to disc. You will need to re-enter them after each login.

UDL Data Mode

UDL tags every elset record with a data classification mode. The session-wide data mode can be set from the UDL panel after connecting:

| Mode | Meaning |
| --- | --- |
| REAL | Operational live data (default) |
| SIMULATED | Synthetic scenario data |
| EXERCISE | Training exercise data |
| TEST | Development/test data |

Two per-panel data mode overrides are also available:

• Threat Sweep — a filter dropdown in the results panel hides rows whose cached TLE was fetched under a different data mode. This is a display-time filter; no re-fetch occurs.

• Intercept Engine — an 'Intercept Data Mode' dropdown on the intercept form re-fetches the red and blue TLEs from UDL under the selected mode immediately before computing.

TLE Source Provider

UDL aggregates TLEs from multiple source providers (e.g., 18 SDS, LeoLabs). When you connect, SPECTRE automatically discovers available sources for your account by probing the ISS elset. A 'TLE Source' selector appears in the UDL panel. Leave it blank to accept UDL's default, or select a specific provider to pin all subsequent TLE fetches to that source.

Setting Scenario Time

Use the Scenario Time panel to define the analysis window. Enter start and stop times in ISO 8601 format (e.g., 2026-03-15T00:00:00Z). If no scenario time is set, SPECTRE defaults to the current UTC time plus 24 hours.

SECTION 4

Operator Dashboard Walkthrough

The main dashboard uses a three-column force grid layout, designed for rapid situational awareness.

Blue Forces (Left Column)

• Lists all friendly blue assets added to the session.

• Each asset displays its object name, NORAD catalogue number, and TLE epoch.

• Add assets by NORAD ID or by pasting a raw TLE.

• Remove assets with the × button.

Red Tracks (Centre Column)

• Lists threat objects being tracked.

• Same add/remove workflow as blue assets.

• Red tracks are colour-coded with a red accent border.

Intelligence & Planning (Right Column)

• Manoeuvre search configuration and intercept engine controls.

• Orbital event detection (apogee, perigee, node crossings).

• Intercept results with per-burn ΔV breakdown.

• Trade-space scatter plot (when multiple solutions exist).

• Session activity log with real-time streaming.

SECTION 5

Managing Blue Assets & Red Tracks

Adding a Satellite

There are three modes for adding a satellite to your session:

• UDL Fetch — Enter the NORAD catalogue number and SPECTRE fetches the latest TLE from UDL automatically. Requires an active UDL connection.

• Catalogue Search — Search the on-orbit catalogue by name or NORAD ID. Select from results to add with a single click.

• Manual TLE — Paste a two-line or three-line TLE directly into the text area. Useful for objects not yet in the public catalogue.

Removing a Satellite

Click the red × button next to any asset or track to remove it from the session. This action is immediate and does not require confirmation. Removing a satellite does not affect previously computed intercept results.

HRR Watchlist

The HRR (High Rate Revisit) Watchlist sub-tab in the Assets panel provides instant access to the Joint Capability Office (JCO) HRR satellite list from UDL. Click 'Fetch HRR' to load the latest notification (up to 3 days lookback). The panel shows two tables — Blue HRR and Red HRR — each with:

| Column | Description |
| --- | --- |
| Actions | One-click → Blue and → Red ingestion buttons (first column) |
| Name | Common name from HRR notification |
| SATNO | NORAD catalogue number |
| Country | Owner country code |
| Orbit | Orbit regime (LEO / MEO / GEO / HEO) |
| Rank | JCO threat rank (0–5, lower = higher priority) |

Clicking → Blue fetches the TLE from UDL and adds the satellite as a blue asset in one action. Clicking → Red does the same as a red track. A confirmation badge replaces the button immediately on success. The table supports column sorting by clicking any header.

HRR objects are also automatically available as target groups in the Threat Sweep panel, grouped by side (Blue/Red) and rank.

SECTION 6

Setting Scenario Time Window

The scenario time window defines the temporal bounds for all orbital event detection and manoeuvre planning. It is set in the Scenario Time panel at the top of the Blue Forces column.

| Field | Format | Default |
| --- | --- | --- |
| Start Time | ISO 8601 (e.g. 2026-03-15T00:00Z) | Current UTC time |
| Stop Time | ISO 8601 (e.g. 2026-03-16T00:00Z) | Start + 24 hours |

Tip: For conjunction analysis, set the scenario window to span at least two orbital periods of the target object to capture all relevant geometry.

SECTION 7

Classical Orbital Transfers

SPECTRE implements three fundamental orbital transfer methods as its classical solver set. Each has distinct trade-offs between ΔV cost, transfer time, and geometric constraints. Understanding these trade-offs is essential for selecting the optimal intercept strategy.

Lambert Transfer

The Lambert problem is the boundary-value problem of two-body orbital mechanics: given two position vectors and a time of flight, determine the orbit connecting them. SPECTRE uses the Izzo algorithm (2015) — a robust, Householder-iteration solver with guaranteed convergence.

• Inputs: departure position, arrival position, time of flight.

• Output: required velocity vectors at departure and arrival.

• The ΔV is the difference between the required velocity and the satellite's current velocity at the departure point.

• Supports both short-way and long-way (multi-revolution) transfers.

Lambert transfers are the most flexible method — they work for arbitrary geometries and arbitrary transfer times. However, they may require higher ΔV than specialised two-burn transfers when the orbits are nearly coplanar.

Hohmann Transfer

The Hohmann transfer is the minimum-energy two-impulse manoeuvre between two coplanar circular orbits. It uses an elliptical transfer orbit that is tangent to both the initial and final orbits.

• Burn 1 (departure): Raise/lower apogee to touch the target orbit.

• Burn 2 (arrival): Circularise at the target altitude.

• Transfer time is exactly half the period of the transfer ellipse.

The vis-viva equation governs the velocity at any point on a Keplerian orbit:

v² = μ (2/r − 1/a)

where μ is the gravitational parameter, r is the radial distance, and a is the semi-major axis.

Bi-elliptic Transfer

The bi-elliptic transfer uses three impulses and two intermediate ellipses. It is more fuel-efficient than Hohmann when the ratio of the final to initial orbit radius exceeds approximately 11.94.

• Burn 1: Boost to a high intermediate apogee (above both orbits).

• Burn 2: At the intermediate apogee, adjust the perigee to match the target orbit.

• Burn 3: At the new perigee, circularise into the target orbit.

The bi-elliptic transfer trades increased transfer time for reduced ΔV. For orbit ratio r₂/r₁ > 11.94, it is always cheaper than Hohmann. Between 11.94 and ~15.58, the savings depend on the intermediate apogee altitude.

Method Comparison

| Property | Lambert | Hohmann | Bi-elliptic |
| --- | --- | --- | --- |
| Number of burns | 1 (single impulse) | 2 | 3 |
| ΔV efficiency | Variable | Optimal for r₂/r₁ < 11.94 | Optimal for r₂/r₁ > 11.94 |
| Transfer time | User-specified | Fixed (half period) | Longer than Hohmann |
| Geometry constraint | None (arbitrary) | Coplanar circular | Coplanar circular |
| Best use case | General intercept | Altitude change | Large altitude ratio |
| Plane change | Included in ΔV | Not included | Not included |

SECTION 8

Tactical Manoeuvres

Beyond classical orbital transfers, SPECTRE provides tactical manoeuvres designed for space control operations. These are computationally lightweight, operationally focused, and optimised for rapid threat response rather than textbook optimal transfers.

Phasing Orbit

The phasing manoeuvre adjusts the satellite's orbital period so that it arrives at the target's angular position after a specified number of revolutions. It is the primary tool for along-track rendezvous when both objects share a similar orbit.

• Burn 1: Lower/raise the orbit to change the period (enter phasing orbit).

• Coast: Complete N revolutions in the phasing orbit, closing the angular gap.

• Burn 2: Return to the original orbit (symmetric with Burn 1).

• More revolutions = lower ΔV but longer wait time.

Operational use: stealth rendezvous, delayed intercept, constellation spacing, passive intercept geometry. The phasing orbit is invisible to observers who only track the manoeuvre burns — the coast phase appears ballistic.

Relative Motion (Hill / Clohessy-Wiltshire)

The CW equations describe linearised relative motion near a circular reference orbit. SPECTRE uses them to compute precise impulses for controlled proximity operations.

Radial Separation

• A single radial impulse produces oscillating radial displacement.

• x(t) = (Δvx / n) sin(nt) — radial position at time t.

• Also creates along-track drift: y(t) = -(2Δvx / n)(1 - cos(nt)).

• Use case: rapid defensive spacing, inspection distance changes, collision avoidance.

Along-Track Drift

• A single along-track impulse produces secular (growing) along-track displacement.

• y(t) = Δvy (4sin(nt) - 3nt) / n — secular term dominates over time.

• Use case: satellite shadowing, constellation phasing, passive intercept.

Combined Manoeuvre

• Solves the full 2×2 CW system for simultaneous radial + along-track targets.

• Operator specifies: 'Create X km radial separation and Y km along-track offset in T minutes.'

CW manoeuvres are most accurate when the relative separation is small compared to the orbital radius (< 100 km). For larger separations, use Lambert or Hohmann transfers instead.

Plane Change

Many tactical intercepts require orbital plane alignment before a transfer can be executed. SPECTRE evaluates two strategies:

• Pure plane change: ΔV = 2v sin(Δi/2). Evaluated at both the equatorial node and apogee — apogee is cheaper for eccentric orbits.

• Combined altitude + plane change: Uses the cosine rule to fold the inclination change into a Hohmann-like transfer. Always cheaper than performing them separately.

Key question for operators: 'Is it cheaper to change plane first, or during the transfer?' SPECTRE answers this automatically by comparing pure and combined costs.

J2 Drift Planner

Earth's oblateness (J2) causes the right ascension of the ascending node (RAAN) to precess secularly. SPECTRE exploits this for fuel-free orbital plane alignment.

dΩ/dt = -3/2 · n · J2 · (R_E / p)² · cos(i)

• Prograde orbits (i < 90°): RAAN drifts westward (negative rate).

• Retrograde orbits (i > 90°): RAAN drifts eastward (positive rate).

• Polar orbits (i = 90°): No RAAN drift — J2 cannot help.

• Different altitudes produce different drift rates → natural convergence.

SPECTRE computes the natural convergence time between two satellites and the small altitude change that would accelerate convergence — trading a tiny ΔV for days or weeks of waiting time.

J2 drift is extremely powerful for strategic orbit planning. A 50 km altitude change can shift the differential RAAN rate by several tenths of a degree per day — potentially saving weeks of waiting for natural alignment.

Collision Avoidance (COLA)

When a conjunction is predicted, SPECTRE computes the minimum ΔV to move the satellite outside the collision probability envelope. Three strategies are evaluated simultaneously:

• Radial kick — displace the satellite perpendicular to its orbit plane.

• In-track drift — along-track impulse creates growing separation via differential period.

• Out-of-plane burn — normal impulse shifts the orbit plane slightly.

SPECTRE selects the cheapest strategy automatically. The operator sees all three options and their ΔV costs in the result notes.

For typical LEO conjunctions with 1+ hour warning, along-track burns are usually cheapest. For short-notice events (< 30 minutes), radial or out-of-plane burns may be the only viable option.

Optimal Defensive Evasion

The evasion planner extends COLA into a full defensive manoeuvre generator. Given an incoming threat, a fuel budget, and available warning time, SPECTRE evaluates multiple burn strategies across multiple timing offsets to find the optimal evasion burn.

• Respects fuel budget constraints — will not exceed available ΔV.

• Evaluates prograde, normal, and radial burns at 25%, 50%, 75%, and 100% of available warning time.

• Reports remaining fuel after the manoeuvre.

• Selects the strategy that achieves the required miss distance with minimum ΔV.

The key difference from COLA: evasion respects operational constraints. A COLA computation may suggest a burn that exceeds your fuel budget. The evasion planner will find the best achievable miss distance within your constraints.

GEO Drift Orbit

For geostationary satellites, SPECTRE computes east-west longitude relocation via drift orbits. A small SMA change induces a secular longitude drift, and a symmetric burn at the target longitude re-circularises the orbit.

dλ/dt = -3/2 · n_geo · Δa / a_geo

• Raising SMA by ~1 km → westward drift of ~0.01°/day.

• Two symmetric burns: enter drift orbit + stop at target longitude.

• Also supports graveyard orbit transfers (GEO + 300 km, standard end-of-life disposal).

GEO drift is extremely fuel-efficient — relocating 30° in longitude over 30 days costs only a few m/s of ΔV. This makes it ideal for strategic repositioning of GEO defence assets.

Natural Motion Circumnavigation (NMC)

The NMC solver computes relative orbits for proximity operations. In CW dynamics, bounded (non-drifting) relative motion follows a 2:1 ellipse: the along-track amplitude is always twice the radial amplitude.

• Passive safety: if propulsion fails, the inspector naturally drifts away from the target.

• Safety margin = radial amplitude (minimum distance from target).

• Single impulse to establish the relative orbit (radial + optional cross-track).

• Used by NASA, ESA, and DoD for formation flying and satellite inspection.

The passive safety guarantee is critical for high-value assets. If the inspector's propulsion fails during proximity operations, a properly designed NMC trajectory ensures it drifts harmlessly away rather than colliding with the target.

Manoeuvre Classification Engine

Given two TLEs of the same satellite at different epochs, SPECTRE estimates what manoeuvre occurred between them. This transforms SPECTRE from a planning tool into a space intelligence tool.

• Compares Keplerian elements (Δa, Δe, Δi, ΔRAAN) between epochs.

• Subtracts expected J2 RAAN drift to isolate manoeuvre-induced changes.

• Classifies: altitude change, plane change, phasing, station-keeping, combined, or unknown.

• Estimates ΔV magnitude and burn direction (prograde, normal, radial, combined).

• Reports confidence level (0–100%) for the classification.

Intercept Detectability Metric

For any computed intercept, SPECTRE assesses how detectable the manoeuvre would be by ground-based space surveillance networks. This enables counter-space analysis and operational security planning.

• ΔV categorisation: micro (< 1 m/s), small (1–10 m/s), medium (10–100 m/s), large (> 100 m/s).

• Detection probability based on altitude regime (LEO/MEO/GEO) and ΔV magnitude.

• Estimated time to detection (hours) — how quickly ground tracking would notice.

• Overall observability score (0–100) combining all factors.

Micro-manoeuvres (< 1 m/s) in GEO are extremely difficult to detect — detection probability below 10% and potentially days before orbital changes become apparent. This is why GEO drift manoeuvres are so strategically significant.

Intent Predict

The intent predictor scores adversary intercept intent by correlating observed orbital behaviour against known attack profiles. It combines manoeuvre classification output with geometric opportunity analysis to produce a composite threat score.

• Analyses recent manoeuvre history (Δa, Δi, ΔRAAN sequence) for convergent behaviour towards a blue asset.

• Geometric opportunity score: evaluates phase angle closure rate, coplanarity trend, and relative altitude convergence.

• Intent score (0–100): weighted combination of behavioural correlation and geometric opportunity.

• Thresholds: 0–25 nominal, 25–50 ambiguous, 50–75 suspicious, 75–100 hostile.

Intent prediction is inherently probabilistic — it identifies patterns consistent with intercept preparation, not certainty. Always combine with intelligence context and orbital geometry before escalating.

Intercept Envelope

The intercept envelope solver computes the probabilistic reachability volume for a satellite within a given time horizon. It sweeps time of flight values across the vis-viva equation to determine the set of orbits achievable within a ΔV budget.

• Vis-viva TOF sweep: evaluates reachable semi-major axes for ΔV budgets from 0.01 to 5.0 km/s.

• Produces altitude-vs-time contour showing the reachability envelope.

• Identifies which blue assets fall within the adversary's reachable set at each time step.

• Supports both prograde-only and arbitrary-direction burn assumptions.

The intercept envelope answers the key operational question: 'Which of our assets can this threat reach, and how quickly?' Use it for force protection prioritisation and early warning.

Stability Analysis

The stability analyser evaluates the boundedness of relative motion between two co-orbital objects using the CW drift condition. Bounded (non-drifting) relative motion requires the initial along-track velocity offset to satisfy:

Δvy₀ = −2n · Δx₀

• Computes the stability score: ratio of actual Δvy₀ to the bounded-motion requirement.

• Score near 1.0 = bounded (formation-stable); score ≫ 1.0 or ≪ 1.0 = drifting apart or converging.

• Reports the secular drift rate in km/orbit if the condition is not satisfied.

• Use case: assess whether an adversary is passively co-orbiting or actively maintaining proximity.

A stability score near 1.0 for an adversary satellite is a strong indicator of deliberate station-keeping relative to your asset — this is a key input to the intent predictor.

Manoeuvre Fingerprinting

The fingerprint engine extends manoeuvre classification by matching observed behaviour against six canonical manoeuvre profiles. Each profile captures a distinct operational pattern with characteristic orbital element signatures.

• Profile 1 — Station-keeping: periodic small Δa corrections, stable inclination and RAAN.

• Profile 2 — Phasing: periodic Δa oscillations with monotonic phase-angle closure.

• Profile 3 — Plane alignment: progressive Δi or ΔRAAN changes toward a target plane.

• Profile 4 — Intercept approach: combined altitude and plane convergence toward a specific target.

• Profile 5 — Evasion: abrupt manoeuvre followed by divergent trajectory relative to a threat.

• Profile 6 — Disposal: monotonic altitude decrease or GEO graveyard insertion.

• Returns the best-matching profile, correlation score (0–1), and confidence level.

Fingerprinting is most effective when multiple TLE epochs are available (3+ manoeuvre observations). Single-epoch classification reverts to the standard manoeuvre classification engine.

Formation Defence

The formation defence solver extends COLA by adding a formation spacing constraint. When defending a formation of blue assets, collision avoidance manoeuvres must not only avoid the threat but also maintain the minimum required spacing between formation members.

• Input: formation member positions, minimum inter-satellite spacing, and the conjunction threat.

• Evaluates COLA manoeuvres that maintain formation geometry within tolerance.

• If no single-burn solution preserves formation spacing, recommends coordinated multi-satellite burns.

• Reports post-manoeuvre formation integrity score and any spacing violations.

Formation defence is critical for distributed space architectures where losing formation geometry may be as costly as the conjunction itself. Always verify formation integrity after any COLA manoeuvre.

Orbital Terrain Mapping

The orbital terrain mapper produces an altitude-vs-inclination risk map characterising the operational environment. It combines three risk layers to produce a composite terrain score for any orbital regime.

• Debris density: spatial density of tracked debris objects per altitude/inclination bin.

• Congestion: number of active satellites and manoeuvring objects in each bin.

• Radiation: trapped proton and electron flux from the AP-8/AE-8 radiation belt models.

• Composite terrain score (0–100): weighted sum of debris, congestion, and radiation risk.

• Use case: route planning for orbital transfers, identifying safe parking orbits, risk-aware manoeuvre selection.

The 800–1000 km altitude band at high inclination (sun-synchronous) typically scores highest for debris risk due to historical fragmentation events. GEO scores highest for congestion. MEO radiation belts peak near 20,000 km altitude.

Min-Time Intercept

The min-time intercept solver finds the shortest time of flight that achieves intercept within a given ΔV budget. It uses a binary search on TOF, evaluating Lambert solutions at each step to find the minimum transfer time that satisfies the fuel constraint.

• Input: blue asset, red track, maximum ΔV budget (km/s).

• Binary search converges on TOF to within 1-second precision.

• Reports the minimum achievable TOF, required ΔV, and the corresponding Lambert solution.

• If no solution exists within the ΔV budget, reports the minimum ΔV required and the corresponding TOF.

Min-time intercept answers the urgent operational question: 'How fast can we get there with the fuel we have?' Use it for time-critical threat response where speed matters more than fuel efficiency.

Complete Method Comparison

| Method | Burns | Best Use Case | ΔV Character |
| --- | --- | --- | --- |
| Lambert | 1–2 | General intercept, arbitrary geometry | Variable |
| Hohmann | 2 | Altitude change, coplanar circular | Minimum energy |
| Bi-elliptic | 3 | Large altitude ratio (r₂/r₁ > 11.94) | Lower than Hohmann for large ratios |
| Phasing | 2 | Along-track rendezvous, delayed intercept | Very low (period adjustment) |
| CW Radial | 1 | Defensive spacing, inspection geometry | Very low (metres/s) |
| CW Drift | 1 | Shadowing, constellation spacing | Very low (metres/s) |
| Plane Change | 1–2 | Inclination alignment | High (proportional to Δi) |
| J2 Drift | 0–1 | RAAN alignment, strategic planning | Near-zero (natural precession) |
| COLA | 1 | Conjunction avoidance | Minimum for required miss distance |
| Evasion | 1 | Defensive manoeuvre under fuel constraints | Budget-constrained minimum |
| GEO Drift | 2 | GEO longitude relocation | Very low (mm/s to m/s) |
| NMC | 1 | Proximity ops, formation flying | Very low (relative orbit establishment) |
| Manoeuvre Detect | 0 | Space intelligence, behaviour analysis | N/A (analytical) |
| Detectability | 1–2 | Counter-space analysis, OPSEC | Same as underlying intercept |
| Intent Predict | 0 | Adversary intent scoring, threat assessment | N/A (analytical) |
| Intercept Envelope | 0 | Reachability analysis, force protection | Budget-swept (vis-viva) |
| Stability Analysis | 0 | Relative motion boundedness assessment | N/A (analytical) |
| Fingerprint | 0 | Behavioural classification against 6 profiles | N/A (analytical) |
| Formation Defence | 1–N | COLA with formation spacing constraint | Minimum for miss + spacing |
| Orbital Terrain | 0 | Risk mapping: debris, congestion, radiation | N/A (analytical) |
| Min-Time Intercept | 1–2 | Fastest intercept within ΔV budget | Budget-constrained minimum TOF |

SECTION 9

Threat Sweep

The Threat Sweep performs a rapid batch intercept assessment of all HRR objects in a chosen target group against your primary blue asset. It runs a Hohmann screening pass across five orbital epochs for each target, then refines the top-five results with Lambert solvers.

Operator Workflow

• 1. Connect to UDL and fetch the HRR watchlist (Assets → HRR tab).

• 2. Open the Threat Sweep hero tab.

• 3. Select a Target Group from the dropdown (e.g. 'Red HRR – Rank 1').

• 4. Click Fetch TLEs — SPECTRE retrieves TLEs for all objects in the group and displays a readiness badge (N/M TLEs ready).

• 5. Click Sweep Targets — the batch assessment runs and the ranked results table appears.

• 6. Optionally click Refine on any row for a full Lambert refinement of that target.

Sweep Algorithm

For each target in the selected group, SPECTRE computes Hohmann ΔV at five canonical epochs: T+0 (now), apogee passage, perigee passage, ascending node, and descending node. All N×5 entries are ranked by ΔV; the five unique lowest-cost targets are then refined with Lambert solvers for precision ΔV and miss distance.

| Result Column | Description |
| --- | --- |
| Target | Satellite name and NORAD ID |
| Orbit | Orbit regime compatibility badge |
| ΔV (km/s) | Minimum Lambert transfer cost (refined) |
| Miss Distance (km) | Closest approach distance at intercept epoch |
| Epoch | Optimal manoeuvre start epoch |
| Method | Screening method (Hohmann/Lambert) |

Data Mode Filter

A 'Data Mode' dropdown above the results table filters displayed rows by the UDL data mode tag of the cached TLE (REAL / SIMULATED / EXERCISE / TEST). Select 'All' to show every target regardless of data mode. This is a display-time filter — no re-fetch occurs.

Orbit regime compatibility is enforced: a LEO blue asset will only sweep LEO targets; a GEO blue asset will only sweep GEO targets. Incompatible targets are excluded with an explanatory note.

SECTION 10

Running Intercept Calculations

Method Selection

Select the intercept method from the dropdown in the Intercept Engine panel. Methods are grouped into two categories:

• Classical Transfers: Lambert, Hohmann, Bi-elliptic, Proximity, Rendezvous.

• Tactical Manoeuvres: Phasing, CW Radial, CW Drift, Plane Change, J2 Drift, COLA, Evasion.

• Advanced Analysis: GEO Drift, NMC (safety ellipse), Manoeuvre Detect, Detectability.

• Decision Support: Intent Predict, Intercept Envelope, Stability Analysis, Fingerprint, Formation Defence, Orbital Terrain, Min-Time Intercept.

The form adapts to show relevant fields for each method — time of flight is shown for Lambert and CW methods; target distance is shown for CW, COLA, and Proximity methods.

Parameters

| Parameter | Description | Default |
| --- | --- | --- |
| Red Satellite | Target threat track to intercept | (select from red tracks) |
| Blue Satellite | Friendly asset performing the manoeuvre | (select from blue assets) |
| Manoeuvre Start | UTC epoch for the first burn | Current UTC time |
| Coast Hours | Coast duration before first burn (hours) | 1.0 |
| Intercept Hours | Time of flight for Lambert solver (hours) | 6.0 |
| Max ΔV | Upper bound on acceptable ΔV (km/s) | 3.0 |
| Intercept Data Mode | Re-fetch red/blue TLEs from UDL under this data mode before computing. Leave blank to use session TLEs. | Session default (REAL) |

Interpreting the Burn Table

The result displays a per-burn breakdown table with the following columns:

• Burn # — sequential burn index (1-based).

• Segment — descriptive name of the manoeuvre segment.

• Burn Epoch — UTC time of the impulsive burn.

• ΔV km/s — total delta-V magnitude for this burn.

• Prograde — velocity-direction component (VNB frame).

• Normal — orbit-normal component (VNB frame).

• Radial — co-normal/radial component (VNB frame).

VNB Reference Frame

SPECTRE decomposes all ΔV vectors into the VNB (Velocity-Normal-Binormal) frame, also known as the local orbital frame. This is the standard frame used by spacecraft operators for manoeuvre planning:

• V (Prograde): Along the velocity vector. Positive prograde raises the orbit on the opposite side.

• N (Normal): Perpendicular to the orbital plane (angular momentum direction). Used for plane changes.

• B (Radial/Binormal): Completes the right-hand triad. Points generally away from the central body.

SECTION 11

Trade-Space Analysis

The trade-space scatter plot visualises all intercept solutions computed during the current session. It plots ΔV (km/s) against transfer time (minutes), with points colour-coded by method:

• Lambert / Proximity — Ops Blue (#00659E)

• Hohmann / Rendezvous — Command Gold (#C8A415)

• Bi-elliptic — Green (#27AE60)

• Tactical manoeuvres (Phasing, CW, Plane Change, J2 Drift, COLA) — distinct colours per method

Using the Trade-Space Plot

Run intercept calculations with different methods and parameters. After the second solution, the trade-space chart appears automatically below the burn table. Hover over any point to see the method, ΔV, transfer time, and miss distance.

Look for the Pareto front — solutions in the lower-left region of the chart offer the best combination of low ΔV and short transfer time. Solutions closer to the axes represent trade-offs: low ΔV with longer transfers, or fast transfers with higher ΔV.

Clearing History

Click the 'Clear History' button on the chart to reset all accumulated solutions and start a fresh trade-space analysis. This also clears the current intercept result display.

SECTION 12

Scenario Planning

Access Windows

SPECTRE detects orbital events — apogee, perigee, ascending node, and descending node — for both red and blue satellites within the scenario time window. These events define natural manoeuvre opportunities.

• Apogee burns are efficient for lowering the perigee.

• Perigee burns are efficient for raising the apogee.

• Node crossings indicate plane-change opportunities.

Conjunction Detection

When both a red track and a blue asset are loaded, SPECTRE can identify close-approach windows by propagating both TLEs through the scenario period. Events are displayed as clickable badges that auto-populate the manoeuvre start time field.

Operator tip: Use orbital events to identify the optimal manoeuvre epoch, then run Lambert solvers at those epochs to minimise ΔV.

SECTION 13

Decision Engine & Scenario Evaluation

The SPECTRE Decision Engine applies game-theoretic analysis to space control scenarios. Operators define a grid of adversary courses of action (COAs) against a set of friendly response options, and the engine evaluates every combination using deterministic SGP4 propagation and a weighted composite scoring function. The result is a ranked response recommendation with an expected utility score and risk-adjusted analysis.

The Decision Engine panel is accessible from the Intelligence column of the main dashboard. It is designed for commander-level use after the initial threat assessment is complete — it answers the question 'Given what the adversary might do, what is our best response?' rather than 'How do we intercept?'

Adversary Action Modelling

The operator defines up to four adversary actions, each representing a distinct course of action the adversary could take. Each action is characterised by:

| Field | Description |
| --- | --- |
| Action ID / Name | Short identifier and descriptive label |
| Action Type | MANOEUVRE, SENSOR_RETASK, POSTURE_CHANGE, or NO_ACTION |
| Probability | Operator-assessed likelihood [0–1] that this COA is executed |
| Confidence | Intelligence confidence in the probability estimate [0–1] |
| Description | Free-text tactical description for context |

The probability and confidence fields allow the operator to express both the likelihood of an action and the quality of the intelligence supporting that estimate. Low-confidence high-probability assessments receive reduced weighting under the probabilistic selector strategy.

Friendly Response Options

Up to four friendly response options are defined in parallel. Each response describes what the blue force would do in reaction to the adversary action:

| Field | Description |
| --- | --- |
| Response ID / Name | Short identifier and descriptive label |
| Action Type | MANOEUVRE, SENSOR_RETASK, POSTURE_CHANGE, or NO_ACTION |
| Cost | Normalised resource cost [0–1] — propellant, ops tempo, personnel |
| Reversibility | How easily the action can be undone [0–1]; 1 = fully reversible |
| Time to Execute (h) | Latency from decision to execution in hours |
| Description | Tactical description of the friendly action |

Scoring Weights

The composite outcome score for each (adversary action, friendly response) pair is computed from five weighted components. The default weights reflect a balanced space control posture; operators can adjust them before running the evaluation:

| Weight | Default | Meaning |
| --- | --- | --- |
| w_custody | 0.35 | Penalty for losing track custody of the adversary asset |
| w_closest_approach | 0.25 | Reward for maximising closest-approach distance |
| w_dv_cost | 0.20 | Penalty for friendly ΔV expenditure |
| w_time_to_exec | 0.10 | Penalty for slow response latency |
| w_reversibility | 0.10 | Reward for selecting reversible, de-escalatory actions |

Increasing w_custody above 0.5 reflects a high-value asset protection posture where losing track of the adversary is unacceptable. Increasing w_reversibility reflects a de-escalation preference — the engine will favour manoeuvres that can be recalled or reversed if the threat assessment changes.

Selector Strategies

After the outcome matrix is computed, SPECTRE applies a selector strategy to identify the recommended response:

| Strategy | Logic | Best For |
| --- | --- | --- |
| MINIMAX | Minimise worst-case composite score across all adversary actions | Conservative — guarantees a floor on worst-case outcome |
| MAXIMIN | Maximise worst-case benefit (minimum gain across all actions) | Optimistic — focuses on preserving upside in best case |
| EXPECTED_VALUE | Minimise expected composite score weighted by adversary action probabilities | Probabilistic — optimal when probability estimates are trustworthy |

The MINIMAX strategy is the default and is appropriate when intelligence confidence is low and the operator wants to hedge against the worst plausible adversary action. Switch to EXPECTED_VALUE when high-confidence probability estimates are available from all-source intelligence.

Output & Recommended Response

The Decision Engine produces an outcome matrix table showing the composite score for every (adversary, friendly) cell. Lower scores are better for the friendly force. The recommended response is highlighted with its strategy label and rationale. Key outputs include:

• Recommended response — the friendly COA selected by the chosen strategy.

• Robust strategy label — which decision rule was applied (MINIMAX / MAXIMIN / EXPECTED_VALUE).

• Per-adversary ranking — for each adversary action, the friendly responses are ranked best-to-worst.

• Computation time — confirms the engine ran to completion.

The Decision Engine output is a recommendation, not an order. Operators retain full authority over response selection. The engine is a structured decision-support tool — it enforces analytical rigour and surfaces non-obvious trade-offs, but context, rules of engagement, and commander intent always take precedence.

SECTION 14

Training Environment

SPECTRE includes a fully sandboxed Training Environment that allows operators to develop and maintain proficiency without touching operational data. The training mode uses synthetic TLEs and scenario parameters entirely; no real satellite tracking data is used or accessible within a training session.

Access the Training Environment by clicking the Training button (shown as a lightning bolt icon) in the navigation bar. Training sessions are isolated from the main operational workspace — any assets, results, or UDL data from the operational session remain unaffected.

Level-Based Progression

Operators progress through five levels, each unlocking more complex scenarios and analysis tools. Advancement requires accumulating points on the relevant skill axes and meeting specific unlock conditions:

| Level | Title | Unlock Condition | Focus Area |
| --- | --- | --- | --- |
| 1 | Observer | Default — available on first login | Basic TLE handling, orbital events, Lambert transfers |
| 2 | Analyst | Complete the introductory tutorial | Threat assessment, manoeuvre classification, PoL basics |
| 3 | Planner | 100+ points in manoeuvre_planning axis | Tactical manoeuvres, COLA, evasion, trade-space analysis |
| 4 | Commander | 5 scenarios completed + 50+ decision_quality points | Decision Engine, multi-threat scenarios, formation defence |
| 5 | Strategist | 3 challenge scenarios completed | Full capability — all methods, gauntlet challenges, adversary modelling |

Skill Axes

Points earned in training are distributed across skill axes, providing a multi-dimensional view of operator proficiency:

| Skill Axis | Description |
| --- | --- |
| manoeuvre_planning | Accuracy and efficiency of orbital transfer and manoeuvre solutions |
| threat_assessment | Quality of threat identification, classification, and prioritisation |
| decision_quality | Soundness of response selection in decision-engine scenarios |
| situational_awareness | Speed and accuracy of orbital event detection and conjunction analysis |
| system_proficiency | Breadth of SPECTRE features used correctly across a session |

Scenario Types

Three scenario types are available, each testing a different operational skill set:

• Analyst scenarios — present a threat object and a TLE history; the operator must correctly classify the manoeuvre type, estimate the ΔV, and assess intent. These scenarios develop threat_assessment and situational_awareness.

• Planner scenarios — provide a blue asset and a threat object; the operator must select the optimal manoeuvre method and parameterise it correctly to meet mission objectives within a fuel budget. These develop manoeuvre_planning.

• Challenge scenarios (Gauntlet) — multi-stage scenarios that combine threat assessment, manoeuvre planning, and decision support. All tools are available; points are awarded for optimal solutions across each stage. Gauntlets are only accessible at Level 5.

Points System

Points are awarded per completed objective within a scenario. Objectives have a base point value and may include a speed multiplier — completing the objective in under half the allotted time doubles the point award. Axis weights determine how base points are distributed across skill axes:

• Correct manoeuvre method selection — base 50 pts, 80% manoeuvre_planning, 20% system_proficiency.

• ΔV within 5% of optimal — base 100 pts with speed multiplier, 100% manoeuvre_planning.

• Correct threat classification — base 75 pts, 80% threat_assessment, 20% situational_awareness.

• Decision Engine response selection matching recommended COA — base 100 pts, 100% decision_quality.

Tutorial completion is a prerequisite for Level 2 unlock. The introductory tutorial walks through adding assets, fetching TLEs from the synthetic UDL endpoint, running a Lambert intercept, and reading the burn table. It takes approximately 10 minutes and can be re-run at any time from the Training menu.

SECTION 15

NOTSO Correlation

NOTSO (Notice to Space Operators) messages are formal notifications issued by spacecraft operators to USSPACECOM prior to planned manoeuvres, proximity operations, launches, or deorbits. SPECTRE's NOTSO Correlation engine parses these free-text messages, temporally correlates them with manoeuvres detected by the Pattern of Life engine, and builds operator behavioural profiles that inform intent assessment.

The correlation analysis answers a fundamental intelligence question: does this operator file NOTSOs before manoeuvring? If so, how reliably and how far in advance? Operators who consistently file precise NOTSOs exhibit predictable, transparent behaviour; those who never file NOTSOs for detected manoeuvres are classified as stealth operators — a significant intelligence indicator.

Parsing NOTSO Messages

NOTSO data may be sourced from the UDL notification endpoint (if available for the operator's account) or pasted directly into the NOTSO panel as free text. Both paths produce structured NOTSO records. The parser handles the standard USSPACECOM template format (field: value line structure) and reasonable variants, extracting:

• Message ID and issuing entity.

• NORAD catalogue number (from SATNO, NORAD ID, or COSPAR fields).

• Effective window: start and end UTC datetime (defaulting to a 24-hour window if end is not specified).

• NOTSO type: MANOEUVRE, DEORBIT, LAUNCH, PROXIMITY_OPS, TEST, or OTHER (inferred from free text).

• Predicted ΔV magnitude (km/s) if stated in the message.

• Predicted burn direction if stated.

Correlation Logic

Each NOTSO is matched against PoL-detected manoeuvres for the same NORAD ID using a configurable tolerance window (default 24 hours):

• Matched — the manoeuvre epoch falls within [NOTSO effective_start − tolerance, NOTSO effective_end + tolerance]. If multiple manoeuvres qualify, the one closest to the NOTSO window midpoint is selected.

• NOTSO-only (phantom) — a NOTSO was filed but no corresponding manoeuvre was detected in the TLE analysis. May indicate a cancelled manoeuvre, a manoeuvre below the ΔV detection threshold, or a false notification.

• Manoeuvre-only (silent) — a manoeuvre was detected but no NOTSO was filed. This is the most intelligence-significant outcome: the operator executed a manoeuvre without notification.

| Metric | Description |
| --- | --- |
| Time offset (h) | NOTSO effective_start minus manoeuvre epoch — positive = NOTSO led the manoeuvre |
| Magnitude ratio | Predicted ΔV from NOTSO divided by detected ΔV — close to 1.0 indicates accurate prediction |
| Window accuracy | Fraction of matched manoeuvres that fell inside the NOTSO window (not just the tolerance band) |

Operator Behaviour Profiles

The correlation results are aggregated into an OperatorBehaviourProfile for each analysed NORAD ID. The profile captures notification reliability, timing patterns, and predictability:

| Profile Field | Description |
| --- | --- |
| Notification rate | Fraction of detected manoeuvres with a corresponding NOTSO [0–1] |
| Mean lead time (h) | Average hours between NOTSO effective_start and detected manoeuvre epoch |
| Lead time std (h) | Standard deviation of lead time — measures timing consistency |
| Phantom NOTSO count | NOTSOs filed with no detected manoeuvre — may indicate deception |
| consistent_notifier | Flag: notification rate > 90% |
| stealth_operator | Flag: notification rate < 30% — significant intelligence indicator |
| predictable_timing | Flag: lead time standard deviation < 6 hours — highly regular behaviour |

A stealth_operator classification (rate < 30%) combined with an increasing intent score from the Intent Predictor is a high-confidence composite indicator of adversarial activity. Escalate immediately to command for all-source intelligence corroboration. Never rely on NOTSO behaviour alone as a basis for operational response.

SECTION 16

Photometry Analysis

SPECTRE's Photometry Analysis module detects anomalous brightness changes in space objects from ground-based optical observations. Brightness anomalies may indicate attitude changes, panel deployments, physical damage, or reflectivity alterations — all of which can be correlated with manoeuvres detected by the Pattern of Life engine to provide a richer characterisation of object behaviour.

The pipeline accepts CSV-format light-curve data, applies geometric and atmospheric corrections, fits a solar-phase-angle baseline using iterative sigma-clipping, and compares a recent observation window against the baseline using a two-sample Welch's t-test. All computations are performed in pure Python with no external scientific library dependency.

Data Ingestion

Photometric observations are provided as a CSV file with the following column structure:

| Column | Required? | Description |
| --- | --- | --- |
| epoch_utc | Yes | ISO 8601 observation timestamp (UTC) |
| apparent_magnitude | Yes | Observed brightness in magnitudes |
| uncertainty | No (default 0.05) | 1-sigma magnitude measurement uncertainty |
| filter_band | No (default 'unfiltered') | Photometric filter (V, R, unfiltered, etc.) |
| range_km | No (default 1000) | Observer–object distance for range normalisation |
| solar_phase_angle_deg | No (default 45°) | Sun–object–observer angle for phase correction |
| elevation_deg | No (default 45°) | Object elevation above horizon for airmass correction |
| lunar_phase_fraction | No (default 0.0) | Lunar illumination [0=new, 1=full] for quality flagging |
| lunar_separation_deg | No (default 90°) | Angular separation from Moon for quality flagging |

Geometric Corrections

Before baseline fitting, each observation is corrected for two geometric effects that would otherwise mask intrinsic brightness changes:

• Range normalisation — apparent magnitude is reduced to a standard range (default 1,000 km) using: reduced_mag = apparent_mag − 5·log10(range/ref_range).

• Atmospheric extinction — Rozenberg (1966) airmass is used below 10° elevation for accuracy; the correction is relative to zenith: Δmag = k·(airmass − 1).

• Quality flagging — observations near the lunar limb (separation < 20° or phase > 0.8) are flagged 'lunar'; low-elevation observations (< 15°) are flagged 'low_elevation'; both together produce 'reject' flags.

Baseline Fitting & Change Detection

The analysis fits a quadratic solar-phase-angle model (mag = a₀ + a₁·phase + a₂·phase²) to the corrected observations using iterative 3σ sigma-clipping. This establishes the object's nominal brightness as a function of viewing geometry. The baseline is then used to compute per-epoch residuals.

Change detection compares residuals from the recent window (default last 30 days) against residuals from the historical baseline window (default 30–120 days ago). A Welch two-sample t-test determines statistical significance:

| Output Field | Description |
| --- | --- |
| baseline_mean / baseline_std | Mean and std of residuals in the historical baseline window |
| recent_mean / recent_std | Mean and std of residuals in the recent window |
| magnitude_change (Δmag) | recent_mean − baseline_mean; positive = object is fainter |
| change_direction | 'brightening' (Δmag < 0), 'fading' (Δmag > 0), or 'none' |
| significant_at_95 / significant_at_99 | Whether the change is statistically significant at 95% / 99% confidence |
| p_value | Two-tailed p-value from the Welch t-test |
| estimated_change_epoch | First epoch where the residual exceeded 2σ of the baseline scatter |
| confidence | Composite confidence score [0–1] based on p-value and sample sizes |

Manoeuvre Correlation & Interpretation

If a statistically significant brightness change is detected, SPECTRE automatically searches for PoL-detected manoeuvres within ±48 hours of the estimated change epoch. Any correlated manoeuvre epochs are reported in the assessment output.

• Brightening correlated with a manoeuvre may indicate panel deployment, attitude reorientation to a more reflective aspect, or removal of a protective cover.

• Fading correlated with a manoeuvre may indicate panel loss or damage, attitude change to a less reflective aspect, or insertion into shadow.

• Brightness change without a correlated manoeuvre may indicate a passive attitude evolution, tumbling onset, or environmental damage not yet large enough to produce a detectable TLE change.

Photometry is a complementary intelligence layer. A confident brightening detection (p < 0.01) that correlates with a NOTSO-silent manoeuvre (stealth operator profile) and an elevated Intent Predict score represents a high-confidence composite indicator of adversarial activity. Treat each layer as corroborating evidence, not proof in isolation.

SECTION 17

Operator Scenarios

Scenario 1: Quick-Look Conjunction Assessment

Objective: Rapidly assess whether a conjunction event requires a manoeuvre.

• Add the blue asset by NORAD ID (e.g., 25544 for ISS).

• Add the red track by NORAD ID.

• Set the scenario window to cover the predicted conjunction epoch ± 12 hours.

• Run Orbital Events to identify the closest approach geometry.

• Run a Lambert intercept at the conjunction epoch to determine the ΔV required for a collision avoidance manoeuvre.

• If ΔV < 0.01 km/s, a small adjustment is sufficient. If ΔV > 0.1 km/s, consider alternative timing.

Scenario 2: Defensive Repositioning of Blue Asset

Objective: Move a blue asset to a safer orbit to avoid a persistent threat.

• Add the blue asset and identify its current orbital elements from the TLE.

• Determine the desired target altitude (e.g., raise by 50 km).

• Run a Hohmann transfer to compute the two-burn sequence for the altitude change.

• Review the burn table: verify the total ΔV is within the satellite's remaining fuel budget.

• Note the transfer time — if too long, consider a Lambert solution with a shorter time of flight.

Scenario 3: Multi-Method Intercept Comparison

Objective: Compare Lambert, Hohmann, and bi-elliptic solutions for the same target pair.

• Add one blue asset and one red track.

• Run Lambert with 6-hour time of flight.

• Run Hohmann for the same satellite pair.

• Run Bi-elliptic for the same pair.

• The trade-space chart appears after the second run. Compare all three points.

• Select the solution on the Pareto front that best balances ΔV and transfer time for your operational constraints.

Scenario 4: Historical TLE Epoch Analysis

Objective: Analyse how intercept costs change with different TLE epochs.

• Add the red track using the most recent TLE from UDL.

• Run a Lambert intercept and note the ΔV and miss distance.

• Remove the red track and re-add with an older TLE (e.g., from a previous day).

• Run the same Lambert intercept and compare results.

• Larger miss distances with older TLEs indicate orbital prediction degradation. Use this to assess TLE freshness requirements for your mission.

Scenario 5: Full Mission Planning Workflow

Objective: Plan a complete intercept mission from initial assessment through solution selection.

• Log in and connect to UDL.

• Set the scenario window to the mission planning period.

• Add all relevant blue assets and red tracks.

• Run Orbital Events for each red/blue pair to identify manoeuvre windows.

• For each promising window, run Lambert, Hohmann, and Bi-elliptic solutions.

• Use the trade-space plot to identify the optimal solution.

• Record the selected burn table (epoch, ΔV components) for uplink to the spacecraft.

• Clear history and repeat for the next target pair if required.

Scenario 6: CW Defensive Spacing

Objective: Create a safe standoff distance from a threat using CW proximity manoeuvres.

• Add the blue asset and the red track (both in similar orbits).

• Select CW Radial Separation with a target distance of 5 km and a time of 30 minutes.

• Review the single-impulse ΔV — typically metres per second for short-range proximity operations.

• If along-track displacement is also needed, switch to CW Along-Track Drift and specify the desired drift distance.

• Compare both CW solutions in the trade-space plot — radial manoeuvres are faster but oscillatory; along-track drift is secular (growing).

Scenario 7: COLA Conjunction Response

Objective: Respond to a predicted conjunction by computing the cheapest avoidance manoeuvre.

• Add the blue asset and the red threat object.

• Run Orbital Events to confirm the conjunction geometry and timing.

• Select COLA and set the required miss distance (e.g., 5 km).

• SPECTRE evaluates all three strategies (radial, in-track, out-of-plane) and recommends the cheapest.

• Review the result notes to see all three strategy costs and the selected option.

• If time permits (> 1 hour to conjunction), the in-track burn is usually cheapest. For urgent conjunctions, radial or cross-track may be the only option.

Scenario 8: GEO Longitude Relocation

Objective: Relocate a GEO satellite 15° eastward over 15 days.

• Add the GEO satellite as a blue asset.

• Add a reference satellite at the target longitude as a red track (or use a GEO satellite already at the desired slot).

• Select GEO Drift and set coast hours to represent the desired drift duration in days.

• Review the two-burn solution: enter drift orbit (small prograde burn) and stop at target longitude (symmetric retrograde burn).

• Verify total ΔV is within the satellite's remaining fuel budget — typical GEO relocations cost only a few m/s.

Scenario 9: Threat Behaviour Analysis

Objective: Classify an observed manoeuvre by an adversary satellite.

• Obtain two TLEs for the same satellite at different epochs (before and after suspected manoeuvre).

• Add the earlier TLE as a red track and the later TLE as a blue asset.

• Select Manoeuvre Detect — SPECTRE compares the orbital elements and classifies the manoeuvre.

• Review: manoeuvre type (altitude change, plane change, phasing, etc.), estimated ΔV, burn direction, and confidence level.

• Use the classification to assess intent — altitude changes suggest repositioning, plane changes suggest target tracking.

Scenario 10: Adversary Intent Assessment with Formation Defence

Objective: Assess adversary intent toward a blue formation and compute a coordinated defensive response.

• Add all formation members as blue assets and the suspected adversary as a red track.

• Run Intent Predict to score the adversary's intercept intent against each formation member.

• Run Intercept Envelope to determine which formation members fall within the adversary's reachable set.

• Run Stability Analysis on the adversary-to-nearest-blue pair to assess whether the adversary is actively station-keeping relative to the formation.

• Run Fingerprint on the adversary's recent TLE history to classify its behavioural profile.

• If intent score exceeds 50, run Formation Defence to compute a coordinated COLA manoeuvre that preserves formation spacing.

• Review the Orbital Terrain map to identify safe retreat orbits with low debris and congestion risk.

• If immediate response is required, run Min-Time Intercept to find the fastest achievable defensive repositioning within the fuel budget.

SECTION 18

Glossary of Terms

| Term | Definition |
| --- | --- |
| Apogee | The point in an orbit furthest from the central body. |
| Bi-elliptic Transfer | A three-burn orbital transfer using two intermediate ellipses. |
| COLA | Collision Avoidance — minimum-ΔV manoeuvre to avoid a predicted conjunction. |
| Conjunction | A close approach between two space objects. |
| Coast | A ballistic (unpowered) phase between manoeuvres. |
| CW Equations | Clohessy-Wiltshire (Hill) equations for linearised relative motion near a circular orbit. |
| ΔV (Delta-V) | Change in velocity; the fundamental currency of orbital manoeuvres. |
| Detectability | The likelihood that a manoeuvre will be observed by space surveillance networks. |
| Drift Orbit | An orbit with a slightly different SMA than GEO, causing east-west longitude drift. |
| Fingerprinting | Behavioural classification of a satellite's manoeuvre history against canonical operational profiles. |
| Formation Defence | Collision avoidance manoeuvre planning that maintains minimum inter-satellite spacing within a formation. |
| Epoch | A specific point in time, usually UTC. |
| Evasion | A defensive manoeuvre to avoid an incoming threat while respecting fuel constraints. |
| GEO | Geostationary Earth Orbit — circular orbit at 42,164 km radius with 24-hour period. |
| Graveyard Orbit | Disposal orbit 300 km above GEO for end-of-life satellites. |
| Hohmann Transfer | A minimum-energy two-burn transfer between coplanar circular orbits. |
| Impulse | An instantaneous change in velocity (idealised thruster firing). |
| Intent Assessment | Scoring of adversary intercept intent by correlating observed behaviour with attack profiles and geometric opportunity. |
| Intercept Envelope | The probabilistic reachability volume of a satellite within a given ΔV budget and time horizon. |
| J2 | The dominant zonal harmonic of Earth's gravitational field, caused by equatorial bulge. |
| J2 Drift | Exploitation of J2-induced RAAN precession for fuel-free orbital plane alignment. |
| Lambert Problem | The two-point boundary-value problem: find the orbit connecting two positions in a given time. |
| Manoeuvre Classification | Estimating the type and magnitude of a manoeuvre from observed orbital element changes. |
| Min-Time Intercept | The shortest time-of-flight intercept achievable within a given ΔV budget, found by binary search on TOF. |
| Miss Distance | The closest approach distance between two objects at the intercept epoch. |
| NMC | Natural Motion Circumnavigation — passive relative orbit for inspection/proximity operations. |
| NORAD ID | Five-digit catalogue number assigned to each tracked space object. |
| Orbital Terrain | Altitude-vs-inclination risk map combining debris density, congestion, and radiation environment. |
| Passive Safety | Property of a relative orbit where natural motion prevents collision if propulsion fails. |
| Perigee | The point in an orbit closest to the central body. |
| Phasing Orbit | A temporary orbit with a different period, used to close along-track separation over N revolutions. |
| Plane Change | A manoeuvre that alters the orbital inclination or RAAN. |
| Prograde | In the direction of the velocity vector. |
| RAAN | Right Ascension of the Ascending Node — the angle from the vernal equinox to the ascending node. |
| Retrograde | Opposite to the direction of the velocity vector. |
| Semi-major Axis | Half the longest diameter of an elliptical orbit. |
| SGP4 | Simplified General Perturbations model 4 — the standard TLE propagator. |
| Stability Score | Ratio of actual Δvy₀ to the CW bounded-motion requirement (Δvy₀ = −2n·Δx₀); 1.0 = bounded relative motion. |
| TLE | Two-Line Element set — the standard format for satellite orbital data. |
| Time of Flight | Duration of the transfer from departure to arrival. |
| Trade-Space | The set of feasible solutions plotted by their key trade-off parameters. |
| UDL | Unified Data Library — authoritative source for TLE data and space object notifications. |
| UDL Data Mode | Classification tag on UDL records: REAL (operational), SIMULATED, EXERCISE, or TEST. |
| VNB Frame | Velocity-Normal-Binormal local orbital reference frame. |
| Vis-viva Equation | Relates orbital velocity to position and semi-major axis. |
| GCAT | General Catalogue — Jonathan McDowell's authoritative space object catalogue at planet4589.org. |
| HRR | High Rate Revisit — JCO satellite watchlist identifying high-priority objects by threat rank. |
| Pattern of Life (PoL) | Analysis of a satellite's full TLE history to characterise manoeuvre behaviour and detect anomalies. |
| PoL Status | Classification of a satellite's historical behaviour: NOMINAL (routine station-keeping), ANOMALOUS (unusual or large manoeuvres), or SUSPICIOUS (evasive or targeting behaviour). |
| TLE Source | The UDL data provider (e.g. 18 SDS, LeoLabs) whose elsets are used for TLE fetches. |
| Drift Phase | A continuous period of east-west longitude drift at a measurable rate, detected from successive GEO elsets. |
| NOTSO | Notice to Space Operators — formal pre-manoeuvre notification filed with USSPACECOM by spacecraft operators. |
| Phantom NOTSO | A NOTSO filed but for which no corresponding manoeuvre was detected in TLE analysis. |
| Consistent Notifier | Operator behavioural flag: notification rate above 90% — reliable pre-manoeuvre notification behaviour. |
| Stealth Operator | Operator behavioural flag: notification rate below 30% — significant intelligence indicator. |
| Predictable Timing | Operator behavioural flag: lead-time standard deviation below 6 hours — highly regular notification pattern. |
| Decision Engine | SPECTRE's game-theoretic scenario evaluation module; computes outcome matrices for adversary × friendly COA grids. |
| MINIMAX | Decision strategy that selects the friendly response minimising the worst-case composite score across all adversary actions. |
| MAXIMIN | Decision strategy that selects the response maximising the worst-case benefit. |
| Expected Value | Decision strategy that selects the response minimising the expected composite score weighted by adversary action probabilities. |
| Composite Score | Weighted sum of custody, closest-approach, ΔV cost, time-to-execute, and reversibility metrics — lower is better for blue force. |
| Photometry | Measurement of the brightness of a space object over time; used to detect attitude changes, physical alterations, or reflectivity anomalies. |
| Reduced Magnitude | Apparent magnitude corrected for range and atmospheric extinction; enables like-for-like comparison across different observing geometries. |
| Phase Function Baseline | Quadratic model of brightness vs solar phase angle fitted to historical observations; the reference against which changes are detected. |
| Welch t-test | Two-sample t-test that does not assume equal variance; used by SPECTRE to compare recent vs historical photometric residuals. |
| Training Environment | SPECTRE's sandboxed training mode using synthetic data; supports five progression levels from Observer to Strategist. |
| Skill Axis | One of five proficiency dimensions tracked in the Training Environment: manoeuvre_planning, threat_assessment, decision_quality, situational_awareness, system_proficiency. |
| Gauntlet | Multi-stage challenge scenario in the Training Environment, accessible at Level 5 (Strategist) only. |

SECTION 19

Historical Pattern of Life Analysis

The Historical Pattern of Life (PoL) panel analyses the full TLE history of a satellite to detect past manoeuvres, characterise station-keeping behaviour, and identify anomalous activity. It fetches up to 5,000 historical elsets from UDL and combines them with the latest current elset to produce a comprehensive orbital element timeline.

Operator Workflow

• 1. Open the Historical PoL hero tab.

• 2. Enter the SATNO to analyse.

• 3. Adjust the ΔV Detection Threshold (default 2.0 m/s) — lower values detect smaller station-keeping manoeuvres; raise it to focus on significant repositioning events.

• 4. Choose the data source: UDL session (uses active credentials) or enter UDL credentials directly.

• 5. Click Analyse — SPECTRE fetches TLE history and the current elset, merges and sorts them chronologically, then runs the PoL engine.

• 6. Review the results: manoeuvre timeline, orbital element charts, drift phase analysis, and PoL status.

Results Panels

| Panel | Description |
| --- | --- |
| Summary | Total TLE count, manoeuvre count, PoL status (NOMINAL / ANOMALOUS / SUSPICIOUS) |
| Manoeuvre Timeline | Table of detected manoeuvres with epoch, estimated ΔV (m/s), manoeuvre type, and longitude at time of manoeuvre |
| Orbital Elements Chart | Time-series plots of altitude, inclination, eccentricity, RAAN, and period across the full TLE history |
| GEO Longitude Chart | Longitude and drift rate over time for GEO/near-GEO objects, with drift phase bands |
| Drift Phases | Table of distinct longitude drift phases: start/end epoch, direction, drift rate (°/day), start/end longitude |
| Statistics | Mean ΔV, 2σ high-ΔV threshold, mean manoeuvre interval, and interval bounds |

Manoeuvre Classification

| Type | Description |
| --- | --- |
| station_keeping | Small periodic manoeuvre to maintain orbital slot or altitude |
| plane_change | Change in inclination or RAAN |
| repositioning | Large manoeuvre indicating a change of operational orbit or longitude slot |
| unknown | Manoeuvre detected but type cannot be classified from TLE data alone |

PoL analysis requires at least 10 valid TLEs for meaningful results. Objects with sparse TLE histories will produce limited manoeuvre detection. The dual-fetch strategy (historical + current) ensures the most recent elset is always included even when the 5,000-record cap is reached.

SECTION 20

GCAT Browser

The GCAT (General Catalogue) Browser provides instant access to Jonathan McDowell's authoritative space object catalogue datasets, hosted at planet4589.org. 28 datasets spanning launch history, payload attributes, orbital decay records, and deep-space catalogues are available.

Accessing the Browser

Open the GCAT Browser hero tab. The 28 dataset tiles load immediately. Click any tile to fetch and display that dataset in a searchable, sortable, paginated table. No UDL connection is required — data is fetched directly from planet4589.org on demand.

Available Dataset Categories

| Category | Example Datasets |
| --- | --- |
| Launch catalogue | Satcat, Launch log, Launch sites |
| Payload attributes | Payload catalogue, Owner/operator, Masses |
| Orbital decay | Decay log, Reentry predictions |
| Deep space | Deep space catalogue, Lunar missions, Planetary missions |
| Debris & fragmentation | Fragmentation events, Debris catalogue |
| Auxiliary | Rocket body catalogue, Infrastructure |

Search and Navigation

Each dataset table supports:

• Text search — filters all visible columns in real time.

• Column sort — click any header to sort ascending/descending.

• Pagination — navigate large datasets in pages of 50 rows.

GCAT data is fetched on demand and cached for the session. Datasets are not updated automatically — click the dataset tile again to refresh.

SECTION 21

Appendices

A. Keyboard Shortcuts & UI Tips

| Action | How |
| --- | --- |
| Quick-add satellite | Enter NORAD ID and press Enter |
| Run intercept | Click Apply Intercept or press Enter in the form |
| View orbital events | Click Compute Events with satellites selected |
| Clear trade-space | Click Clear History on the scatter plot |
| Switch TLE mode | Use the radio buttons (UDL / Manual / Catalogue) |

B. Common Error Messages

| Error | Cause | Resolution |
| --- | --- | --- |
| No TLE found | Satellite not in session | Add the satellite first |
| Intercept calculation failed | Solver diverged or invalid geometry | Try a longer time of flight or different coast duration |
| UDL connection failed | Invalid credentials or network error | Re-enter UDL credentials; check network connectivity |
| No orbital events found | TLE epoch too stale or window too short | Use a fresher TLE or widen the scenario window |

C. API Endpoints

| Method | Endpoint | Purpose |
| --- | --- | --- |
| POST | /plan/maneuver/apply-intercept | Run an intercept calculation |
| POST | /plan/maneuver/apply-all-intercepts | Run all intercepts (bulk) |
| GET | /plan/maneuver/orbital-events | Compute orbital events for satellites |
| GET | /plan/maneuver/trade-space-data | Retrieve trade-space JSON for charting |
| POST | /plan/maneuver/clear-history | Clear intercept history and reset chart |
| GET | /plan/threat/target-config | Load threat sweep target group dropdown |
| POST | /plan/threat/fetch-targets | Pre-fetch TLEs for a target group |
| POST | /plan/threat/sweep | Run batch Hohmann + Lambert threat sweep |
| POST | /plan/threat/refine | Lambert-refine a single sweep entry |
| GET | /pol/panel | Load the Historical PoL panel |
| POST | /pol/analyse | Run PoL analysis for a SATNO |
| GET | /pol/chart-data/{satno} | Retrieve PoL chart JSON for a SATNO |
| POST | /assets/blue/quick-add | One-click HRR → blue asset ingestion |
| POST | /assets/red/quick-add | One-click HRR → red track ingestion |
| POST | /blue/add | Add a blue asset to the session |
| POST | /red/add | Add a red track to the session |
| POST | /blue/remove | Remove a blue asset |
| POST | /red/remove | Remove a red track |
| GET | /admin/users | List all users (admin only) |
| POST | /admin/users | Create a new user (admin only) |
| GET | /admin/users/{id}/edit | Open inline edit form for a user (admin only) |
| GET | /admin/users/{id}/cancel | Cancel inline edit, return display row (admin only) |
| POST | /admin/users/{id} | Save role / password change for a user (admin only) |
| DELETE | /admin/users/{id} | Delete a user account (admin only) |
| POST | /udl/login | Connect to UDL with credentials |
| POST | /udl/logout | Disconnect from UDL |
| POST | /udl/data-mode | Set session UDL data mode |
| POST | /udl/tle-source | Set preferred TLE source provider |
| GET | /udl/tle | Fetch TLE from UDL by NORAD ID (latest or epoch mode) |
| GET | /udl/statevector | Fetch latest Cartesian state vector from UDL |
| GET | /udl/hrr | Fetch HRR satellite list from UDL |
| GET | /udl/catalog/search | Search cached on-orbit catalogue |

SECTION 22

User Management

SPECTRE provides a web-based user management console at /admin/users. This interface is restricted to accounts with the admin role. Operators do not have access to this area.

Accessing the Admin Console

Navigate to /admin/users in your browser while logged in as an admin. Non-admin accounts receive an HTTP 403 Forbidden response. Unauthenticated sessions are redirected to the login page.

The bootstrap admin account is created from the SPECTRE_ADMIN_USER and SPECTRE_ADMIN_PASS environment variables on first run (only if the users table is empty). Set these before starting the server for the first time.

User Roles

| Role | Permissions |
| --- | --- |
| operator | Full access to all operator console features. Cannot access /admin/*. |
| admin | All operator permissions plus /admin/users account management. |

Creating a User

At the top of the /admin/users page, complete the Add New User form:

• Username — unique login name, up to 64 characters.

• Password — initial password. The account holder should change this on first login.

• Role — select Operator or Admin.

Click Create User. The new account appears immediately in the user table below. A confirmation banner confirms success or describes any validation error (e.g. duplicate username).

Editing a User

Click Edit on any user row to open an inline edit form. Two fields are available:

• Role — change between Operator and Admin.

• New Password — enter a new password to reset it. Leave blank to keep the existing password.

Click Save to apply changes, or Cancel to discard and return to the display row. Username cannot be changed; create a new account and delete the old one if a username change is required.

Deleting a User

Click Delete on any user row. A confirmation prompt is shown before the account is permanently removed. The following constraints apply:

• An admin cannot delete their own account.

• An admin cannot delete or demote the last remaining admin account.

Deletion is permanent and immediate. There is no soft-delete or recycle bin. Ensure the correct account is selected before confirming.

Fallback — Direct Database Access

If all admin accounts are lost (e.g. deleted via direct SQL), access can be restored by stopping the server and either: (a) deleting spectre.db and restarting with SPECTRE_ADMIN_PASS set — this re-runs the bootstrap — or (b) for PostgreSQL, truncating the users table and restarting.